# OA-1K TOLD Mission Planning Tool

DATA PROVENANCE AND DIGITIZATION METHODOLOGY

Prepared for: 185 SOS Standardization/Evaluation
Tool version documented: engine v1.9.9 (September 2026)
Status: PLANNING AID ONLY — all outputs are computed from digitized flight-manual charts and must be verified against TO OA-1K-1 before operational use.

## 1. Purpose and Scope

This document describes how the performance data inside the OA-1K TOLD mission planning tool was generated, how the tool computes its outputs at run time, and how both were validated. The tool is a single, self-contained HTML file that runs offline. Every performance number it produces traces to one of the source documents listed in Section 2; the tool contains no performance data from any other origin and does not extrapolate beyond the printed envelope of any chart.

## 2. Source Documents

| Source | Content used |
| --- | --- |
| TO OA-1K-1, Appendix A (Flight Test Generated Data, 12 Sep 2025; CERTIFIED PTO 30 Jan 2026) | Takeoff (Figs A-1..A-3), climb (A-4/A-5), landing (A-7/A-8), accelerate-stop (A-9..A-11), cruise (A-19..A-28), specific range (A-30..A-39), V-speed and stall tables (Tables A-1..A-8), Section V limits |
| C1F AFM Supplement C0001B.42.001 (Table 6-13 / Fig 6-3) | Center-of-gravity envelope (fwd 25.0 in; aft 35.0 in to 11,400 lb, straight-line to 30.5 in at 16,000 lb), applied against the unit-directed interim MGTOW of 15,190 lb |
| Signed DD 365-F (Form F) records, including the fleet Form F compilation (41 pages) | Per-tail Chart C basic weight and moment for 16 of 22 tails (latest Form F per tail) |
| Unit W&B CG Calculator v1.2.xlsm | Chart C data for the remaining 6 tails; stores item weights/moments; SCL definitions; crew roster; the weight-and-balance computation method itself |

## 3. Chart Digitization Method

The Appendix A charts are vector line drawings. Each chart page was rendered to a 300-dpi image and processed with computer-vision software (OpenCV/scipy, with OCR for printed labels). The process for every chart family followed the same discipline: calibrate, trace, verify.

Calibration. The software locates the chart's axis frame and printed axis labels and establishes a pixel-to-engineering-units mapping. Wherever possible the calibration was anchored to chart structure rather than OCR alone: on the cruise panels, the airspeed axis was locked to the arithmetic grid spacing and cross-checked against the printed VNE reference lines, which read 141 KIAS on every panel — a chart-provided consistency check on the axis mapping of all 123 panels.

Curve tracing. Each printed curve was isolated by line-weight and intensity thresholding and traced pixel-by-pixel into a numeric table. No functional form was assumed; the stored data is the printed geometry itself.

Correction grids as ratio surfaces. The takeoff, landing, and accelerate-stop carpets apply weight, wind, and slope corrections by following diagonal guide lines. Following a guide line by hand multiplies the entry distance by a factor that depends on where the line is entered and how much correction is applied. Each grid was therefore captured as a family of traced guide lines, and the engine interpolates between adjacent traced lines exactly as a pencil interpolates between printed ones. This reproduces the chart's drawn behavior rather than an assumed mathematical model of it.

The cruise and specific-range families. On each of the 123 cruise/SR panels, the four solid gross-weight curves converge into a tight bundle at high airspeed, interleaved with dashed odd-thousand-pound curves. Tracing them independently invites track-jumping, so all seven curves are traced jointly under an order-preserving constraint (curves may not cross or exchange positions), with the dashed curves modeled explicitly so the solids cannot lock onto them. Torque scales were re-anchored to each panel's own printed torque labels; the fuel-flow side scale was fit simultaneously through the printed GAL/HR and LB/HR labels, which merge at 6.7229 lb/gal — itself a check on the scale fit. Each panel's drag-index delta-torque curve and maximum-recommended-torque line were extracted the same way.

Tables. The V-speed tables (VR, unimproved VR, V50, VY, VREF) and stall tables were taken directly from the PDF text layer with no image processing, then visually verified against the printed pages.

Weight-and-balance records. The fleet Form F compilation is scanned (no text layer). Each page was OCR'd at 300 dpi, and every value retained in the tool — tail, date, basic weight, moment — was then visually verified against the page image by direct inspection. Where a tail has multiple Form Fs, the latest date was kept; duplicate pages of the same tail served as independent reads of the same values. For twelve unchanged tails the fleet-PDF values matched the unit workbook exactly, independently confirming both.

## 4. Post-Processing Quality Assurance

Digitized cruise data passed through three automated QA stages. Gross-weight ordering was enforced on every panel. Each panel was compared against its pressure-altitude siblings using a linear-in-temperature trend; cells deviating more than 5% (about 3% of all cells, concentrated in the converged-bundle region) were dropped and refilled from the panel's own remaining weight curves. Panel temperature labels were corrected by aligning the OCR'd sequence against the known ISA-offset progression of the chart series.

## 5. Runtime Computation Model

The engine interpolates linearly between pressure-altitude and temperature panels, quadratically across the printed gross-weight curves, and applies carpet-chart corrections in the same order the printed chart applies them. It never extrapolates: conditions outside any chart's printed envelope return no data rather than an estimate, with a caution to the user. Best-range speed (VRANGE) uses the long-range-cruise definition — the highest airspeed retaining at least 99% of maximum range — which reproduces the manual's printed VRANGE line (validated against the Fig A-18 worked example). Mission fuel integrates the climb chart in 2,000-ft steps at maximum-recommended-torque fuel flow, then cruises the remaining distance at VRANGE or the pilot-selected speed, with a gross-weight refinement pass. Weight and balance replicates the unit workbook's method with per-tail Chart C data, and all TOLD computations draw their weights from that calculation.

## 6. Validation Results

Every chart family was validated against the flight manual's own printed worked examples, run through the digitized engine at the example's stated conditions.

| Check | Printed value | Tool value | Agreement |
| --- | --- | --- | --- |
| Fig A-7 landing example (total over 50 ft) | 3,700 ft | 3,775 ft | 2.0% |
| Fig A-9 accelerate-stop example | 5,000 ft | 5,134 ft | 2.7% |
| Fig A-17 cruise base torque | 2,210 ft-lb | 2,223 ft-lb | 0.6% |
| Fig A-17 DI-92 corrected torque | 2,900 ft-lb | 2,904 ft-lb | 0.2% |
| Fig A-17 fuel flow | 81 gal/hr | 80.8 gal/hr | 0.2% |
| Fig A-18 max-range example (IAS / TAS / FF / TQ) | 138 / 161 / 71 / 2,500 | 138 / 161 / 70.7 / 2,487 | exact / exact / 0.5% / 0.5% |
| Rotate and climb-out speeds (Tables A-1/A-3) | 88 / 96 kt | 88 / 96 kt | exact |
| W&B vs Form F 22-0003 (ZFW, ramp, takeoff wt/mom/CG) | — | — | exact |
| W&B vs unit workbook, full chain, workbook-sourced tail | — | — | exact (all blocks, zero-diff) |

Two independent cross-checks supplement the worked examples. Specific range computed from the digitized cruise panels (TAS divided by fuel flow) agrees with the separately digitized specific-range charts to approximately 3.5% RMS across the envelope. And the printed VNE reference lines and the dual fuel-flow scales, described above, verify the axis calibrations chart-wide.

## 7. Known Limitations and Source-Document Findings

The tool's About & Methods panel carries this list in full; the items below are those a standards reviewer should know.

The Fig A-1 takeoff worked example does not validate to the same tolerance as the other families — the example's entry arrow is drawn at the ISA-curve crossing rather than on the printed 6,000-ft PA curve (finding SD-1). The engine's entry point matches the correct printed curve within 0.4%, and rescaling the printed example to the correct entry agrees with the engine within about 2.5%. Additional findings: the Fig A-4/A-5 climb example arrows are internally inconsistent (SD-2); Fig A-11 is titled "Unpaved Runway" while its conditions block reads "paved" — treated as unpaved per the title (SD-3); Fig A-13 (engine torque for takeoff) resisted reliable automated digitization and its numeric torque-available check is deferred — torque limits are enforced and displayed instead (SD-4); and the Fig A-7 approach-speed axis reads about 1.4 kt above the Table A-5 (1.3 VS) interpolation — the tool uses the printed table (SD-5). One cruise panel (16,000 ft, ISA+10) failed calibration and is bridged by interpolation from its neighbors, and the hottest high-altitude panels carry reduced-span curves reflecting the chart's own gray-zone limits.

## 8. Configuration Control

Each released version is numbered, carries its engine version in the page footer, and is archived; the current version sits at the base level of the unit's tool folder with all superseded versions retained in the Archive subfolder. The About & Methods panel inside the tool documents the engine provenance, method decisions, and source findings for the version in hand.